## 1. What is it used for?

- 3D printers are used to create physical objects from digital designs.

- They are used to make prototypes, models, and product samples.

- They are used to manufacture customized or one-of-a-kind parts.

- They are used to produce tools, replacement parts, and functional components.

- They are used to create complex shapes that traditional manufacturing cannot easily make.

- They are used in education, engineering, medicine, and hobby projects to turn ideas into real items.

## 2. Is it a complex design? Or is it a simple design?

Depends on what you’re making:

Simple designs (boxes, flat shapes):

- easy to print

- fewer supports

- faster and cheaper

Complex designs (overhangs, curves, mechanical parts):

- may require supports

- higher precision or smaller layer height

- better cooling

- slower printing to avoid defects

The complexity affects both printing difficulty and time.

## 3. Is this for prototyping purposes?

If you’re prototyping:

- use fast print settings (large layer height, low infill)

- choose cheaper materials like PLA

- focus on speed, not perfection

If it's the final version:

- use higher quality settings (smaller layer height, strong infill)

- choose materials based on the final use case (PETG, ABS, Nylon)

## 4. How long do I want it to last?

Longevity depends on:

- Material (PLA = short life in heat; PETG/ABS = longer; Nylon = longest)

- Environment (heat, humidity, sunlight)

- Mechanical stress

- Wall thickness + infill

Long-lasting prints usually need stronger materials, higher infill, and thicker walls.

## 5. How tough or brittle should the print be?

Different materials behave differently:

- PLA → stiff but brittle

- PETG → tough and slightly flexible

- ABS → strong and heat-resistant

- Nylon → very tough, flexible, impact-resistant

- TPU → rubber-like and bendable

Your needs decide the material and print settings.

ILabs uses the FDM printer

| Feature | SLS (Selective Laser Sintering) | FDM (Fused Deposition Modeling) | SLA (Stereolithography) |
| --- | --- | --- | --- |
| Printing Material | Polymer powder | Thermoplastic filament (PLA, ABS, PETG, etc.) | Liquid resin |
| Printing Method | Laser fuses powder particles layer by layer | Heated nozzle extrudes melted filament layer by layer | Laser cures liquid resin layer by layer |
| Print Quality / Resolution | High | Lowest | Very high |
| Surface Finish | Slightly rough (powdery texture) | Layer lines visible | Smooth and detailed |
| Strength / Durability | Very strong and durable | Moderate strength (depends on filament) | Strong but can be brittle |
| Accuracy | Excellent | Moderate | Excellent |
| Support Structures | Not required (powder supports the print) | Required for overhangs | Required for most prints |
| Cost (Printer + Materials) | Very expensive | Cheapest | Expensive |
| Ease of Use | Complex (industrial use) | Easy (ideal for beginners) | Moderate (careful handling of resin needed) |
| Speed | Moderate | Fast for small models | Slow to moderate |
| Applications | Functional prototypes, end-use parts, industrial manufacturing | Basic prototypes, hobby models, educational use | Functional prototypes, detailed models, molds, jewelry, dental models |

ILabs 3D printer uses PLA

| Property | PLA (Polylactic Acid) | ABS (Acrylonitrile Butadiene Styrene) | PETG (Polyethylene Terephthalate Glycol) |
| --- | --- | --- | --- |
| Ease of Printing | Very easy to print — low warping, beginner-friendly | Harder to print — prone to warping, needs heated bed/enclosure | Moderate — easier than ABS but trickier than PLA |
| Strength | Moderate strength | High strength and impact resistance | High strength and flexibility |
| Flexibility | Brittle | Moderate | Flexible and tough |
| Heat Resistance | Low (~60 °C softening) | High (~100 °C) | Medium (~80 °C) |
| Durability | Not very durable; degrades in heat and sunlight | Very durable; resistant to wear | Durable and chemical-resistant |
| Surface Finish | Smooth and glossy | Matte finish | Slightly glossy and smooth |
| Environmental Impact | Biodegradable (corn-based) | Non-biodegradable (petroleum-based) | Recyclable but not biodegradable |
| Odor When Printing | Little to no odor | Strong, unpleasant fumes | Mild odor |
| Best Uses | Prototypes, models, educational prints | Functional parts, enclosures, mechanical components | Functional prototypes, food containers, outdoor parts |
| Requires Enclosure? | No | Yes (to prevent warping/fumes) | Not necessary but helpful for consistency |
| Cost | Low | Medium | Medium to slightly high |

| Filament | Pros | Cons |
| --- | --- | --- |
| PLA | Easy to print, biodegradable, good surface finish | Brittle, low heat resistance |
| ABS | Strong, durable, heat-resistant | Warps easily, emits fumes, harder to print |
| PETG | Strong, flexible, durable, chemical-resistant | Can string easily, slower print speed needed |

| Feature | Thermoplastics | Thermosetting Plastics |
| --- | --- | --- |
| Heat behavior | Soften when heated and harden when cooled repeatedly | Harden permanently after first heating; cannot be remelted |
| Recyclability | Can be recycled easily | Cannot be recycled once set |
| Flexibility | Usually soft, flexible, and ductile | Usually hard, rigid, and brittle |
| Molding process | Can be reshaped or remolded multiple times | Can be molded only once |
| Applications | Packaging, bottles, toys, electrical insulation | Switches, circuit boards, kitchenware handles, adhesives |

Nozzle Blocked

- Occur when filament cools in nozzle

- Fixed by pressing the ‘Unload Extruder’ Option

Stringing

- More commonly occur due to lack of support when printing the object at a certain orientation

- Less commonly occur when the filament cools in the nozzle, blocking it halfway

Nozzle Grinding

- Occur when nozzle is too close to the build plate

- Fixed by raising the nozzle by changing the nozzle settings

Warping

- Corners or edges of the print lift or detach from the print bed

- Due to material overcontracting

- Fixed by ensuring the built plate is leveled correctly so that it is not too far from build plate / use adhesion

Make sure 3d printing device is compatible with file type STL (Standard Tessellation Language)

Ultimaker cura 5.2.1

Shelf thickness dont touch about it but it’s about external thickness

Adhesion is stickiness between the print bed and 1st player of the model.

| Feature | Skirt | Brim | Raft |
| --- | --- | --- | --- |
| Touches the model? | ❌ No | ✔ Yes | ✔ Yes (under full base) |
| Adhesion strength | Low | Medium | Very High |
| Primary purpose | Prime nozzle, check flow | Increase surface area, reduce warping | Maximum adhesion, compensate uneven bed |
| Material usage | Very low | Low–Medium | High |
| Removal difficulty | None (not attached) | Easy | Harder; may leave marks |
| Effect on bottom surface | None | Slightly wider base | Rougher bottom, less detail |
| Best for | All prints (general use) | Warping materials, small bases | ABS, nylon, large parts; bad beds |
| Downsides | No adhesion improvement | Adds cleanup time | More printing time + wasted filament |